1. Start up your favorite web browser, which will display your selected homepage.

2. If you are using a proxy (especially a host-based one), disable it if possible to

examine uncached network traffic.

3. Also better to clear browser cache, cookies if you have previously displayed this

page.

4. You may need to disable anti-virus protection software before your own IP address

will show up in captured data.

5. Start up the Wireshark software, select the Capture pull down menu and select

Interfaces.

6. Here we will be using nslookup tool, which is available in most Linux/Unix and

Microsoft platforms today.

9. Command Run 1: nslookup www.gtu.ac.in or www.facebook.com or www.google.com

nslookup www.gtu.ac.in, here this command tells please send me the IP address for the host

www.gtu.ac.in the response from this command provides two pieces of information: (1) the

name and IP address of the DNS server that provides the answer; and (2) the answer itself,

which is the host name and IP address of www.gtu.ac.in. response came from the local DNS

server; it is quite possible that this local DNS server iteratively contacted several other DNS

servers to get the answer.

10. Command Run 2 : nslookup –type =NS www.facebook.com or type nslookup –type =NS

www.gtu.ac.in, here provided the option “-type=NS” and the domain “facebook.com” or

“gtu.ac.in”. This causes nslookup to send a query for a type-NS record to the default local DNS

server. In words, the query is saying, “please send me the host names of the authoritative DNS

for facebook.com, gtu.ac.in

11. ipconfig (for Windows) and ifconfig (for Linux/Unix) command show your current TCP/IP

information, including your address, DNS server addresses, adapter type and so on.

13. we learned that a host can cache DNS records it recently obtained. To see these cached

records,

after the prompt C:\> provide the following command:

ipconfig /displaydns

Each entry shows the remaining Time to Live (TTL) in seconds. To clear the cache, enter

ipconfig /flushdns

Flushing the DNS cache clears all entries and reloads the entries from the hosts file.